Practical Exercise 2 Spatial Transformations & Forward Kinematics Chayada Muangboonsri, Tengo Iluridze

# 1. Overview

This practical exercise we explore spatial transformations and forward kinematics for robotic systems, implemented in Python and CoppeliaSim simulation environment. The lab is structured around three progressive tasks: 1.) converting between different rotation representations, 2.) building a 6-DOF snake robot model in CoppeliaSim , and 3.) computing the full forward kinematic chain programmatically. Together, these tasks bridge the gap between mathematical theory and real simulation practice, showing how rotation matrices, Euler angles, and quaternions are interconnected and how they drive a robot's spatial reasoning.

# 2. Task 1: Rotation Representations

Task 1 focused on implementing conversions between the three main rotation representations used in robotics: quaternions, rotation matrices, and Euler angles. CoppeliaSim internally uses the X-Y-Z extrinsic Euler convention, meaning the total rotation is computed as R = Rx(α) · Ry(β) · Rz(γ). Our job was to implement this logic manually in Python.

We implemented three functions: convert_quaternion_to_matrix, which maps a quaternion [x, y, z, w] as ordered by CoppeliaSim into a 3×3 rotation matrix; euler_from_matrix, which extracts X-Y-Z Euler angles from a rotation matrix using atan2 and asin with a gimbal-lock guard; and matrix_from_euler, which reconstructs the rotation matrix from Euler angles.

This task went relatively smoothly. The main thing to be careful about was CoppeliaSim's quaternion component order (x, y, z, w rather than the w-first convention some libraries use), but once that was confirmed in the API documentation, the implementation followed the standard formulas without major issues.

# 3. Task 2: Building the Snake Robot in CoppeliaSim

Task 2 involved constructing a 6-DOF planar snake robot directly in the CoppeliaSim scene. The robot consists of six revolute joints connected by cuboid links, each 0.25 m long, all lying flat in the XY plane in the zero configuration.

The kinematic structure is captured in the parameter table below. Each row defines the transform from one frame to the next, with columns [Tx, Ty, Tz, Rx, Ry, Rz] followed by the joint type:

| Frame | Tx | Ty | Tz | Rx | Ry | Rz | Joint | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | - | World to Base |
| Joint 1 | 0.25 | 0 | 0 | 0 | 0 | -π/2 | Rz(q1) | Base to J1 |
| Joint 2 | 0.25 | 0 | 0 | 0 | 0 | +π/2 | Rz(q2) | J1 to J2 |
| Joint 3 | 0.25 | 0 | 0 | 0 | 0 | -π/2 | Rz(q3) | J2 to J3 |
| Joint 4 | 0.25 | 0 | 0 | 0 | 0 | +π/2 | Rz(q4) | J3 to J4 |
| Joint 5 | 0.25 | 0 | 0 | 0 | 0 | -π/2 | Rz(q5) | J4 to J5 |
| TCP | 0.25 | 0 | 0 | 0 | 0 | 0.0 | Rz(q6) | J5 to TCP |

While the physical construction of the model was straightforward, the most confusing part was getting the joint orientations right. The joints alternate between −π/2 and +π/2 static offsets in the Z direction. If even one of these offsets is set incorrectly, every subsequent joint frame is misaligned --- the error propagates all the way to the TCP. This cascading effect made debugging a bit tricky, but once we verified each joint offset individually against the expected frame orientation in the simulator, the model fell into place.

# 4. Task 3: Forward Kinematics

Task 3 required computing the Tool Centre Point (TCP) pose (position and orientation) from the joint angles alone, using a programmatic forward kinematics function, and then verifying it against CoppeliaSim's ground truth.

Initially, we misunderstood the task and simply read the TCP orientation and position directly from CoppeliaSim using getObjectOrientation and getObjectPosition, converting the result to degrees to observe the values. This was a useful first step to understand what numbers we were aiming for, but it did not actually implement any kinematics, we were just reading what the simulator already computed.

We then implemented the forward_kinematics function from scratch. It starts with a 4×4 identity matrix representing the world frame, applies the base translation from row 0 of the parameter table, and then iterates over each joint: for each one, it builds the joint rotation Rz(qi) followed by the link geometry transform (translation followed by the three static rotations Rx, Ry, Rz), chaining them together via matrix multiplication:

def forward_kinematics(rob, joints):

TCP = t.identity_matrix()

base_translation = t.translation_matrix(rob[0][0:3])

TCP = t.concatenate_matrices(TCP, base_translation)

for i in range(len(joints)):

curr_row = rob[i + 1]

Tx, Ty, Tz, alpha, beta, gamma = curr_row

Rotatejoint = t.rotation_matrix(joints[i], [0, 0, 1])

TransformLink = t.concatenate_matrices(

t.translation_matrix([Tx, Ty, Tz]),

t.rotation_matrix(alpha, [1, 0, 0]),

t.rotation_matrix(beta,  [0, 1, 0]),

t.rotation_matrix(gamma, [0, 0, 1])

)

TCP = t.concatenate_matrices(TCP, Rotatejoint, TransformLink)

return TCP

After computing the TCP matrix, we extracted the upper-left 3×3 rotation block and passed it through euler_from_matrix to obtain the Euler angles in radians, then converted to degrees for comparison. The resulting position and orientation matched CoppeliaSim's output within floating-point precision (differences below 1e-6), confirming that our implementation was correct.

# 5. Summary

This lab gave us a hands-on opportunity to translate rotation mathematics into working Python code and connect it with a real simulation environment through the CoppeliaSim remote API. We implemented quaternion-to-matrix conversion, Euler angle extraction, and a generic forward kinematics chain from first principles — and verified each step against the simulator's own output.

The most valuable takeaway was how small errors compound in kinematic chains, a wrong joint offset in Task 2 or a wrong matrix multiplication order in Task 3 would silently produce plausible-looking but incorrect results. Systematic comparison against a known ground truth. In this case, CoppeliaSim's built-in position and orientation readouts was essential for catching and fixing those mistakes. The resulting forward_kinematics function is fully generic and works for any N-DOF serial chain described in the same parameter format, which makes it a practical foundation for future robot programming work.